In an emergency, every second counts. When you call 999, you are in the worst moments imaginable, and getting you the help you need as soon as possible could be the difference between life and death. It’s those few seconds that count, and that’s why we designed HALO – Helicopter Ambulance for Lifesaving Operations. HALO is a light twin engine coaxial rotorcraft in the 3-4 tonne weight class, with 2 pilots and a capacity for 6 passengers. It's designed from the ground up with the patient and clinician in mind, with a cabin larger than any other helicopter in its class allowing for sufficient space for all necessary equipment for life-saving operations, even during flight.

HALO has a novel take on old ideas; as the first civilian coaxial helicopter since the 1990s, HALO has performance equal or better than any competitor in the light twin market. A cruise speed of 140 kts and max speed of 156 kts brings us to the scene as fast as possible, with a range of 1018 km and an endurance of 3.2 hours allowing us to reach patients who would otherwise not get the care they need. Two contra rotating props provide a maximum climb rate of 12 m/s and a service ceiling of 20,700 ft, getting us to patients wherever they are. The rotors are 3 bladed with a diameter of 10.60 m, with twist and taper distributions custom designed for maximum performance across our mission profile. A bearingless rotor hub design minimises maintenance cost whilst still ensuring full pitch authority and ease of control.

HALO was designed with safety in mind. All design criteria were matched with EASA CS-29 regulations. It has capacity for two pilots, allowing full night-time operation under UK regulations, expanding our possible working time, and providing round the clock critical care anytime, anywhere. Advanced safety systems such as automatic terrain warning and navigational FLIR allow operations into all terrain no matter the visibility or time of day. The lack of a tail rotor reduces risks to bystanders compared to normal tail rotors and allows for a rear loading ramp for faster patient loading times. The design of the cockpit maximises automation and also provides unparalleled visibility for the pilot.

Twin Honeywell HTS900 turboshaft engines power HALO, providing 1.3 MW of power across all flight conditions. A custom transmission system reduces losses, whilst the exhaust jet is used in a state of the art active flow control system. The use of this system reduces parasitic drag by 13-15%, representing fuel savings and an increase in cruise speed and ceiling.

The cabin, as mentioned, provides ample space for clinicians to work in. Consideration was given to the latest in clinical innovations, such as capability for portable ECMO and ultrasound imaging, or space and equipment for REBOA for the most critically unwell patients. The cabin was designed in consultation with experienced ambulance clinicians, giving input into the specific needs of prehospital care.

Whilst composites are fairly common in the civil aviation market, the use of composites in rotorcraft is still new. By using carbon composites in the aircraft skin and rotor blades, significant weight savings are achieved for similar performance, whilst metal alloys are used where appropriate for maintainability and cost. Consideration was also given to the fatigue life of the components, aiming for minimal replacements to further reduce maintenance time and repair costs.

Advanced control systems also keep our aircraft safe and stable across all flight regimes. P and PI controllers are implemented across a 9 dimensional MIMO system, with gain scheduling across all flight conditions.

Whilst designed for the HEMS mission, flexibility was maintained for other markets such as VIP travel, public utility operators and other end markets. Our design has the potential to be adapted for any requirements that the end user needs, such as by trading cabin space for extra fuel and range or different mission equipment for search and rescue.

HALO is one of the only rotorcraft optimised specifically for HEMS operations, which is why we think it is the obvious choice for operators. We want what is best for the patient, and we hope you agree that HALO’s pioneering approach is the best solution.